Laboratory #6:  Conservation of Energy for a Simple Pendulum

Purpose:  (1) To investigate the properties of energy conservation (i.e. the relationship between potential and kinetic energy) associated with the motion of a simple pendulum.  (2) To calculate the potential and kinetic energy associated with a simple pendulum.

Part A:  Calculation of Kinetic and Potential Energy

- An Olympic high diver (mass = 70 kg) stands on a platform 20 m above the surface of the water, what is the diver’s potential energy?

- The athlete in (a) then dives into the water.  If it assumed that all of the diver’s potential energy is conserved, what is the diver’s kinetic energy at the moment just before entering the water?

- What is the diver’s speed just before entering the water?  Solve the problem based on your answers above.

- Determine the average power expended by the gravitational force to pull the diver to the earth.

Part B:  Investigating a Simple Pendulum

A pendulum consists of a mass attached to a string that is fastened so that the assembly can swing in a plane.  For a simple or ideal pendulum, all of the mass is considered to be concentrated at a point at the center of the mass.

When the mass is pulled in one direction, the height of the mass (H) is elevated relative to its height when the mass is freely hanging.  Since the mass has been raised it has acquired potential energy relative to when it is freely hanging (i.e. at its lowest point!).  When the mass is released, it swings during which time it loses potential energy but gains kinetic energy (since the mass gains speed during the downward phase of the swing).  At its lowest point, the kinetic energy of the mass should be the same as potential energy the mass had at its highest point.  This is because the energy of the mass is conserved, as long as there are negligible frictional forces, which might “suck” energy from the system.

Using a plumb bob, some string, and a photogate timer we will construct a simple pendulum and test this hypothesis.

- Weigh the plumb bob using a triple scale.

- Construct a simple pendulum using the plumb bob according to the directions of the instructor.

- Set-up a photogate sensor at the lowest point of the pendulum.  Align it with the bob so that the mass will not hit it during the downswing. The photogate will be used to measure the speed of the mass at the bottom of its swing.

- Using a non-permanent ink marker, mark a location on the bob where the mass crosses the sensor. (This point should be the center of the mass).

- Measure the height of the mark from the tabletop.  This height (ho) will be the baseline height of the pendulum mass.

- Perform a couple of practice swings to verify that the photogate is working and in the correct location.

- Since the diameter of the bob will need to be determined to calculate its speed through the photogate, measure the thickness using a vernier caliper.  Is there more than one diameter?  If so, which of the values is the most useful for determining the bob’s speed?

- Pull the mass to one side.  Using a ruler, bring the mass to a height of roughly 1 cm above its free hanging height (i.e. its lowest point), keeping the string tension tight.  Record this height (h).

- Let the bob swing through the photogate and record the velocity of the bob as it crosses the sensor.

Part C:  Determination of Kinetic & Potential Energy of the Bob

Practice calculation:  By measuring the mass (on a triple beam balance) and velocity (with the photogate) we can calculate the kinetic energy of mass on the pendulum:

kinetic energy = KE = ½ * mass (in kg) * velocity2 (in m/s)

The velocity of the bob is found by swinging the mass across the photogate which will trigger the sensor when it passes through it.  By dividing the thickness of the bob by the photogate reading:

velocity = v = (thickness of bob) / (timer reading)

The LoggerPro software will automatically calculate the speed when the pendulum passes through the photogate (use experiment file “PHY101-Exp07”).  Be sure to enter the value of bob thickness in the Photo Data Collection section of LoggerPro.  Use these equations to practice calculating the kinetic energy of the bob when it moves across the photogate.

The potential energy of the bob can be determined by measuring the height the mass is elevated above its free hanging height (its lowest point!):

potential energy = PE = mass (kg) * g (m/s2) * height (m)

To calculate height of the bob, measure the initial height (ho) when the bob is at its lowest point.  Then, raise the bob to its elevated position (h) and measure this height.  The height value that is used in the potential energy equation is the difference between these two values (H = h-ho)!

Part D:  Conservation of Energy

Conservation of energy means that for an event such as the mass swinging on a simple pendulum the amount of energy before the swing begins (the potential energy) equals the amount of energy at the bottom of the downswing (the kinetic energy).  By measuring the potential energy of the lifted mass before its release and measuring the kinetic energy at the lowest point of the swing ,as in Part C above, we can prove this principle of physics to be true.  Friction and other interference is our enemy in this task.  Thus we will probably need to repeat the measurement at least 3 times to improve the accuracy of this investigation.  This process of repetition and accuracy improvement is the history of experimental physics.

Preliminary Data:

Mass of Bob =  m  =		___________

Acceleration of gravity =  g  =	___________

Thickness of Bob =  D  =		___________

Initial height of Bob = ho = 	___________

Table 1:

| Trial # | Bob Height (h) | H (h-ho) | velocity (v) | PE | KE | % Uncertainty |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |

Using Graphical Analysis make a graph of kinetic energy (KE) vs potential energy (PE) for the simple pendulum.  Using the “Automatic Curve Fit” function, calculate the best-fir line for the data.  Determine the slope of the line and y-intercept for this graph.  Record your results below.

Summary Calculations

Slope of the graph (PE vs KE) = ______________

Y-Intercept of the graph =   ______________

Final Questions

What is the significance of the slope of your graph?

What is the significance of the y-intercept of your graph?

Can you relate your data and graph to the Principle of Energy Conservation?

Directions to Set-up Photogate (One Gate Mode)

- Plug in photogate into DG1 & turn on ULI

- turn-on computer

- select programs ® Vernier Software ® LoggerPro

- In LoggerPro

- Open experiment file “PHY101-Exp07”

- select set-up ® Data Collection

1.  select “photogate timing”

2.  click on “sampling”

- select “Gate Time - One Gate”

- enter thickness of bob

- click “OK”

Notes:

- LoggerPro collects time data from the photogate then calculates speed (v)

- You can create a meter window to view the current speed value.  To do this:

- select “Window”® “New Wide Window”® “Meter”

- A window will pop-up, deselect all of the meters except v1.

- Click “OK”

- Position the meter in a convenient location on the screen.

- To see speed (v) data, make a graph window narrower (from right).  This will expose the “v” column in the data window