Laboratory #2: Density

I.  Purpose

- To perform physical measurements of various substances.

- To use the data to calculate the density of solid & liquid matter

- To identify the substance based on information found in the CRC.

# II.  Theory

Density is a physical property of substances that relates the mass to the volume the object occupies.  The relationship between density, mass and volume is:

density = mass/volume

Under normal circumstances, density is a physical characteristic that can be measured and used to determine the identity of an “unknown” substance.  In this experiment, the density of several solid objects and liquids will be measured using a variety of techniques.  The identities of these substances will be determined by comparing the measured densities with accepted values obtained from the CRC.

length

diameter

radius

See additional information included in procedure below.

III. Procedure:

Part 1:  Density of a “regular” solid.

Obtain a metal cylinder.  Record the identification number and describe its physical characteristics.  Obtain its mass using the centigram balance.  Use the metric caliper to determine the diameter and length of the cylinder.  Record your measurements in the table below.

| ID # | mass | diameter | length | volume | Density |
| --- | --- | --- | --- | --- | --- |
| Identity: |  |  |  |  |  |

Calculate the density of your cylinder using the data collected for Part 1 and identify according to the data you retrieved from the CRC.  Record the density in the table above.

Volume may be calculated according to the following formula:

Vcyl =  * length * (radius)2,

Note:   = 3.14159 and the radius = (diameter /2)

Part 2: Density of an “irregular” solid.

Obtain a sample of irregular solid from the instructor.  Record the identification number of the solid and describe its physical characteristics.  Measure a 25mL sample in a beaker and obtain the mass of the solid.  Pour 20mL of water into a plastic 50mL graduated cylinder and record the exact volume.  Note that the water surface may have a slight downward curve.  Measure from the bottom of the curve (also known as the meniscus).  Tilt the graduated cylinder and slide the metal carefully into the water.  Be certain there is enough water to cover the metal completely.  Read the new volume.  The difference between the initial and final volume readings is equal to the volume of the metal sample.

| ID # | mass | Initial Volume | Final Volume | Volume Displaced | Density |
| --- | --- | --- | --- | --- | --- |
| ## Identity: |  |  |  |  |  |

Use the data obtained in Part 2 to calculate the density of the metal sample and record the density value in the above table

Part 3:  Density of a liquid.

Select an unknown liquid and record the sample number in your notebook.  Observe its characteristics and record.  Pour 2-3 mL of your unknown into a 10mL graduated cylinder.  Record the exact volume.  Weigh the graduated cylinder with the sample, and record this value (do not subtract out the mass of the container).  Repeat at least 3 more times, adding approximately 2 mL more of your unknown liquid to the amounts already in the graduated cylinder.  Your data should include 4 weighings and the volumes corresponding to them.

| ## Mass of cylinder = __________ {measured} |  |  |
| --- | --- | --- |
| Trial # | mass | volume |
| ## Density (slope) = __________ |  |  |
| ## y-intercept = _________ {predicted mass of cylinder} |  |  |
| ## Known Density of water (4oC) = __________ |  |  |

Using Graphical Analysis Software, graph your data, using the appropriate conventions.  Volume should be on the x-axis, mass should be on the y-axis.  Be sure to label your graph appropriately.  Use the software to draw a “best-fit” straight line through the data points on the graph, making certain that the line intersects the y-axis.

The slope of the line will give you the density of the unknown liquid (the units will be g/mL).

The y-intercept will give you the mass of your empty graduated cylinder.

Record the slope and y-intercept of your graph in the above table.  Compare with your data for accuracy.

Question: How does your value for the density of water compare with the density of water at 4oC?

Question:  How does the y-intercept of your graph compare with the measured value of the graduated cylinder (with no liquid)?

Part 4:  The “known” density of various substances.

Use the textbook to locate the densities of the following: lead, gold, copper, aluminum, iron, and brass.

| Substance | Density |
| --- | --- |

Compare the density measurements you obtained in Part 1 & 2 to identify unknown substances.

VI.  Questions:

- An irregular lump of metal weighing 29.34g was claimed to be pure gold.  It was dropped into a graduated cylinder reading 35.3mL of water.  If the sample is indeed pure, what should the second volume reading be?

- Will lead float in liquid mercury?  Use your data to explain.