Experiment: Heat Engines

Objectives:

- To construct a simple heat engine

- To explore the phases of the heat engine

- To calculate the work performed by the heat engine

- To calculate the efficiency of the heat engine

Equipment:

- Low friction glass syringe

- 25 mL Erlenmeyer flask

- small black rubber cork

- stopcock valve

- 3 1000 mL beakers (or comparable size)

- a ring stand with test tube clamp

- small amount of ice

- electric hot plate

- 50 g hanging mass

- meter stick or ruler

- digital thermometer (or temperature probe w/ ULI & computer)

Introduction:

Machines are devices that convert energy into work (or in some cases, one form of work into another).  Heat engines are machines that convert heat energy into mechanical work.  In order for a heat engine to perform it must have a “working substance”, such as a gas or liquid that must absorb heat energy, perform work and discard the remaining energy that cannot be transformed into work.  In this experiment, you will construct a heat engine that will utilize air as its working substance.  Heat energy will be absorbed from a hot water source and work will be performed via a frictionless glass syringe.  A cold water source will serve to absorb the unusable heat energy from the heat engine.

Syringe scale

Added mass

Cold reservoir

Hot

reservoir

Trapped

air inside

Piston

Platform

Flask

Stopcock

valve

Procedure:

- Construct a simple heat engine (as shown on the right):

- Attach glass syringe to ring stand (plunger on top)

- Place stopcock valve onto rubber cork and insert it into a 25 mL Erlenmeyer flask

- With the plunger positioned at about 5 or 6 cc, attach stopcock to syringe opening (there should be room for the plunger to move both up and down as the air inside compresses and expands).

- Fill 1000 mL beaker (or comparable size) ½ full with cold tap water

- Fill 1000 mL beaker (or comparable size) ½ full with hot tap water

- Place flask into cold water.  Observe the syringe.  What happens?

- Place flask into warm water.  What happens?

- Measure temperature of the cold water bath.  Tcold = _______

- Place flask back in cold water.  Place 50 g mass onto syringe plunger.  What happens to the plunger?

- Record the syringe reading.  Position of syringe = _______

- Measure temperature of the warm water bath.  Thot = _______

- Place flask back in warm water.  What is the new level of the syringe the plunger?

- Record the syringe reading.  Position of syringe = _______

- Remove the 50 g mass.  What happens to the syringe?

- Return the flask to the cold water bath.  You have just completed a complete heat engine cycle.  Record all of your above values into the table below.

- Convert all temperature readings to Kelvin and enter these values in the table.

- Add several ice chunks to the cold water bath & ~50 mL of near boiling water to the hot water bath.  Repeat steps (4) to (11) with the colder and warmer water baths.  Record your values in the table below.

Data Table:

| Cycle 1 | Temperature (oC) | Temperature (K) | Syringe Position (y) | y |
| --- | --- | --- | --- | --- |
| Cold water bath |  |  |  |  |
| Warm water bath |  |  |  |  |
| # Cycle 2 | Temperature (oC) | Temperature (K) | Syringe Position (y) | y |
| Cold water bath |  |  |  |  |
| Warm water bath |  |  |  |  |

Final Analysis

- Can you describe or characterize each of the phases of your heat engine?

- How does the temperature difference between Thot and Tcold (T) affect the output of your engine?

- To quantify the output of your heat engine, calculate the work performed by each engine cycle:

Work = W = m * g * y

Where m is the mass of the hanging weight, g = 10 m/s2 (the acceleration of gravity), and y is the change in elevation of the hanging weight.  Calculate the work performed by the 2 engine cycles.  Note:  to determine y, you will need to use a ruler or meterstick and measure the distance from position 1 to position 2 for the plunger (based on your position (volume) readings).

Wcycle1 = ______

Wcycle2 = ______

- Which engine cycle performed the most work?

- For an “ideal” heat engine, the efficiency can be calculated using the following relationship:

% efficiency = % e = (1 – Tcold/Thot).100 %

Calculate the % efficiency of your heat engine for both of your above engine cycles.

% e1 = _________

% e2 = _________

- Which of the engine cycles was the most efficient?

- Do your answers to (4) and (5) agree?  Why or why not?

- Summary your experience during this experiment and what you have learned about heat and heat engines.