## Lab #6:  Specific Heat & Calorimetry

Objective: In this experiment, the properties of heat and temperature will be investigated, to measure the specific heats of common metals.

Required:

- electric hot plate

- specific heat specimens

- 500 or 1000 mL beaker

- tongs

- Styrofoam cups w/ cover

- gram balance

- 2-3 thermometers (Celsius)

- cold deionized water

- CRC Handbook of Chemistry & Physics

Discussion

Have you ever held a hot piece of pizza by the crust only to have the moister parts burn your mouth when you take a bite?  The meats and cheese have a high specific heat capacity, whereas the crust has a low specific heat capacity.  How can you compare the specific heat capacities of different materials?

In this experiment you will increase the temperature of metal specimens in boiling water and then place them in Styrofoam cups or calorimeters that contain a mass of water equal to the specimen at room temperature.  The heat lost by the specimen equals the heat gained by the water.

Qlost = Qgained

msssTs = mwswTw

The specific heat capacity of the specimen is

ss = (mwcwTw) / (msTs)

For water the specific heat capacity (sw) is 1.00 cal/goC.  If the mass of the water is the same as the mass of specimen, then the specific heat of the sample is

ss = 1.00 cal/goC (Tw / Ts)

Procedure

Step 1.  Place a Styrofoam cup inside a second cup.  You have just constructed an inexpensive double-walled calorie meter, called a calorimeter.

Step 2.  Measure the mass of each specimen.  This value will be needed for the following steps.

Step 3.  Insert the specimens into a beaker with cold water and place the beaker onto a hot plate.  Turn on the hot plate and bring the water to boiling temperature.

Step 4.  Select the specimen you are going to use.  Add the same mass of water to your calorimeter as the selected specimen.  Use a graduated cylinder to carefully measure the same amount of water as there are grams for the specimen (Note: 1 mL of water has 1 gram of mass!).  Pour the water into the calorimeter.  Use a thermometer to measure the temperature of the water in the calorimeter.

Step 5.  Boil the specimens in the water for more than a minute until you are convinced that they are in thermal equilibrium with the water.  Measure and record the temperature of the boiling water.  This should be the same temperature as the specimens, since they are in thermal equilibrium.

Step 6.  Using tongs, quickly remove the selected specimen from the boiling water and place it in the calorimeter.  Be sure to shake any droplets of water from the specimen.  Place a cover over the calorimeter to minimize excess heat loss.

Step 7.  When the water temperature inside the calorimeter reaches a stable value, record the final temperature (which should also be the final temperature of the specimen!).

Step 8.  Repeat steps 4-7 for a total of 3 specimen.

Table A:  Your Data

|  |  | Calorimeter |  |  | Specimen |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Specimen | Mass (g) | Tinitial (oC) | Tfinal (oC) | Tw (oC) | Tinitial (oC) | Tfinal (oC) | Ts (oC) |

Analysis

- Calculate the specific heat capacity values for your specimens  (in cal/goC) and record the values in Table B.

- Convert the specific heat capacity values to units of J/goC.  Note:  The units J/goC are the same as J/g.K, since a temperature change in oC is the same as a temperature change in K units.

Table B:  Analysis

| Specimen | s (cal/goC) | s (Joules/goC) | % Error |
| --- | --- | --- | --- |

Summing Up:

1.  Look up the specific heat values for the elements in Table C.  Fill in the corresponding values in the table.

2.  Compare your values for the specific heats to those in Table A and predict the substance of each specimen.  How do your values compare?

3. Convert the specific heat capacity values in Tables B & C from cal/goC to Joules/goC.

4. Determine the % error between the measured value of each specimen and the predicted substance.  Record % error values in Table B.

Table C:  {from pg 4-120 in Specific Heats of Elements @ 25 oC (CRC, 1995-1996)}

| Specific Heat Capacity |  |  |
| --- | --- | --- |
| Substance | s (cal/goC) | s (Joules/goC) |
| Aluminum |  |  |
| Cadmium |  |  |
| Copper |  |  |
| Gold |  |  |
| Iron |  |  |
| Lead |  |  |
| Silver |  |  |
| Tin |  |  |
| Zinc |  |  |
| Mercury |  |  |
| Water |  |  |