Pressure:

1) Determine the liquid pressure for the following: {neglect atmospheric pressure}

- The liquid pressure at a depth of 1 m below the surface of pure water:

mercury

Closed end

(no pressure)

Air pressure

H

The liquid pressure at a depth of 10 m below the surface of pure water:

- The liquid pressure at a depth of 10 m below the surface of sea water:

2) The weather man/woman on the local channel states that the barometric pressure is 29.5.  The units for this measurement are inches of mercury.  This measurement refers to the height of a closed column of mercury above the surface exposed to the open air (see diagram).

- What is the height of mercury (H) in meters?

- How would this measurement change if the atmospheric pressure were to increase?

Buoyancy:

You build a boat that has a mass of 50 kg.

1) How much water (minimum) must the empty boat displace in order for it to float in pure water (i.e. river or lake water)?  Express your answer in both L (metric) and m3 (SI).

2) How much water (minimum) must the empty boat displace in order for it to float in salt (sea) water?  Express your answer in both L (metric) and m3 (SI).

3) How much pure water must the boat displace if a 70 kg person stands on the boat while it is floating?

Archimedes’ Principle:

You purchase a “gold” statue of mass 1.5 kg and weight 15 N.

1)  What is the mass of the statue in grams?

2) If indeed that statue is made of gold, how much volume would you expect it to displace when you submerge it in pure water?

3)  Suppose you do submerge the statue in water and the volume of water displaced is 132 mL.  What is the statue like likely made of?  Refer to Table 12.1 for density values.

4)  While the statue is submerged, what is the buoyancy force exerted on it by the water?

5) If you attempted to weigh the statue while it is submerged, what would the scale reading be?

6) Would you expect the scale reading to be different if the statue were made of gold?  Explain.