Ohm’s Law & Electrical Circuits:

Bulb

12 V

+

-

1) Consider electrical circuit shown on the right:

- In what direction does electric current flow through the circuit?  Draw the direction of the current on the diagram (use an arrow).

- In what direction do the electrons flow through the circuit?

- The resistance of the bulb (Rbulb) is 30 .  What is the current (I) flowing through the light bulb?

- What is the power used by the light bulb?

- If the circuit is connected for 60 seconds, how much energy is consumed?

- How many electrons flow across the bulb in 1 second?

Practical Application: A 100 W light bulb is on for 8 hours a day for 30 days.

i) How much energy (in joules) does the light bulb consume during the 30 days?

ii) How much energy (in kW.hr or kilowatt-hour) does the light bulb consume during the 30 days?

iii) Compare this value with your value in (i).  Which value would you prefer to see on your power bill?

iii) The electric company charges about $0.04 per kW.hr.  How much does it cost to use this bulb for the 30 days?

2) A second light bulb with the same resistance is connected in series with the bulb in problem #1.

- Draw the new circuit:

- What is the current that will flow through each of the bulbs?

- How much power does the new circuit use?

3)  The second light bulb is connected in parallel instead of in series with the bulb in problem #1.

- Draw the new circuit:

- What is the current that will flow through each of the bulbs?

- How much current total is drained from the battery?

- How much power does each bulb use?