1. Consider the following heat engine, where steps AB and CD are isobaric and steps BC and DA are adiabatic (yes, the adiabatic steps should have a slight curvature).  Note: the working fluid is 10 moles of ideal gas in this engine.

A

B

C

D

- What is the temperature at each point A-D?

- How much work is performed by the gas during each step?

- What is the net work during the complete cycle?

- How much heat (QH) is absorbed by the gas?

- How much heat (QC) is discarded by the gas?

- What is the efficiency of the heat engine?

# What is the Carnot efficiency for the engine?

2. Consider the following heat engine. Steps AB and CD are isothermic and steps BC and DA are isochoric.  The temperature at CD (TH) is 2500 K and the temperature at AB (TC) 500 K.  Note: the working fluid is 5 moles of ideal gas in this engine.

A

B

C

D

# What is the pressure at each point A-D? Calculate using the ideal gas law.

- How much work is performed by the gas during each step?

# What is the net work during the complete cycle?

- How much heat (QH) is absorbed by the gas?

- How much heat (QC) is discarded by the gas?

# What is the efficiency of the heat engine?

# What is the Carnot efficiency for the engine?

H)  What is the change in entropy (SA-B) during AB?

I)  What is the change in entropy (SC-D) during CD?

J)  What is the net change in entropy (Suniverse) between the steps in H and I?