The Electric Potential and Potential Difference:

1. Consider parallel plate capacitor (air filled ) with a charge of +1.5 C (q1) on each of its plates ( = 1x10-6C/m2) and a plate distance of 1x10-4 m.

-

-

-

-

-

-

-

-

-

+

+

+

+

+

+

+

+

+

a. What is the magnitude of the electric field between the plates?

b. What is the capacitance of the capacitor?

c.  How much energy is stored in the capacitor?

d. What is the energy density for this charged capacitor?

e. If the capacitor were filled with a dielectric material (while still maintaining the same amount of charge on the plates) what is the new capacitance?

2. Point charge problem…

3.  2 Point charge problem

4.  Multiple charges

4.  Insulating sphere

a. What is the magnitude of the surface charge density on each plate?

b.  What is the magnitude of the electric field inside the plates of the capacitor?

c.  What is the electric force exerted on an electron placed inside the plates of the capacitor?

d.  What is the acceleration of the electron while it is in between the capacitor plates?

e.  If the plate separation is 0.2m, what is the final speed of an electron (initially at rest), that is placed at the negative charged plate?

Gauss’ Law:

4.  Consider a hollow spherical conductive shell of radius (R) 0.2 m with a fixed charge of +2.0 mC uniformly distributed on its surface.

R = 0.2 M

Q = +2.0 mC

- What is the electric field at all points inside the sphere? Express your answer as a function of the distance (r) from the center of the sphere.

- What is the electric field outside the sphere? Express your answer as a function of the distance (r) from the center of the sphere.

R = 0.2 M

Q = +2.0 mC

What if the sphere is a solid conductive sphere? What is the electric field at all points inside the sphere? Express your answer as a function of the distance (r) from the center of the sphere.

d)   What is the electric field at all points outside the sphere? Express your answer as a function of the distance (r) from the center of the sphere.

5.  Consider a “line of charge”, where the charge per unit length is  (in C/m) and the line is “infinitely long”.

- Draw the line of charge and attempt to draw the E field lines around the line.

- Apply Gauss’s Law to the line of charge and determine the magnitude of the electric field at all points away from the line? Express your answer as a function of the distance (r) from the line of charge.

- If the surface charge density is 0.5x10-3 C/m, what is the magnitude of the electric field at a distance of 0.2 m from the line of charge.

6.  Consider two concentric conductive spheres (of radius r1 and r2, respectively).  The inner sphere has a uniform surface charge (Q) of +2.0 C.

- Draw in the charged regions of the 2 spheres on the diagram.  (Remember that charged is induced in the conductive outer sphere)

- What is the magnitude of the electric field at the following regions?

r1

r

r2

i)  r < r1

- ii) r2 > r > r1

- iii) r > r2

7. Consider the same parallel plate capacitor in problem 3.

+Q

-Q

+

+

+

+

+

+

+

+

+

-

-

-

-

-

-

-

-

-

a. Apply Gauss’ Law to the capacitor to determine the magnitude of the electric field between the plates.