Experiment: Atwood Machine

OBJECTIVES

- To measure the acceleration in 2 different cases

- To analyze the relationship between F and a

- To study a two body system

- To determine the acceleration due to gravity, g

- To estimate the friction force for an Atwood Machine

MATERIALS

LoggerPro				slotted weights

Force Sensor				Smart Pulley

Accelerometer				photo-gate

Dynamics cart				lab stand

C clamp

According to Newton’s Second Law, the acceleration of an object depends on the net force acting on it and on its mass. All the forces acting on an object can be represented on a free-body diagram. Examples of forces are gravity, friction, tension in a string, etc.

During the first part of this lab, you will prove Newton’s Second law by measuring the net force acting on a cart and its resulting acceleration. During the second part, you will study a 2-body system known as Atwood Machine. By measuring the acceleration of the system, you will determine the free fall acceleration, g, and the friction force present in the system.

PRELIMINARY QUESTIONS

- A variable force in the range 0 to 50 N is applied to an object of mass m=1Kg . Predict and sketch the acceleration vs. force graph.

- How would your graph change if the mass of the object is now 5 Kg? What is the significance of the slope of the graph?

m1

m2

For the system represented in the figure below,

  - Draw the free body diagram for each object, considering the mass of the pulley and the mass of the string to be zero

  - Calculate the acceleration of the system in terms of m1, m2 and f (the friction force).

Part I: Newton’s Second Law

PROCEDURE:

1. Attach the accelerometer to the dynamics cart, so as the arrow will point in the direction of the motion. Attach the Force Sensor to the same cart with the hook toward outside (so you can hold the hook)

2. Connect the Low-g Accelerometer to the Ch 1 and the Dual Range Force Sensor to the Ch2 of the LabPro interface.

3. Open the experiment “09 Newton’s Second Law” from the Physics with Computers directory.

4. Click on the “Zero” icon to zero both sensors and then push “Collect”

5. Grab the hook of the Force Sensor and move the cart back and forth for 10 seconds. The force applied to the hook and the acceleration of the cart will be recorded.

ANALYSIS QUESTIONS

1. Examine the Force vs. time and acceleration vs. time graph using the “ Examine” option under the Analyze pull-down menu. When is the force maximum? What is the value of the acceleration when the force is maximum? What is the maximum value of the acceleration?

2. What is the shape of the Force vs. acceleration graph? Find the best fit.

3. Cut-and-paste the graph, including the best fit information, into Word.

4. What is the significance of the slope of the Force vs. acceleration graph? What are the units?

5. Disconnect the sensors from the interface and find the total mass of the object (cart+sensors) using the digital scale. Enter your result below.

Total mass = _______

6. Compare the mass to the slope of the graph determined in step 3. Write a conclusion regarding the relationship between force, mass and acceleration below

Part II: Atwood Machine

For the system described in the Preliminary Question, the acceleration depends on the masses of the 2 objects and the friction force:

(1)

During this experiment, the sum m1+m2 will remain constant, but the difference m1-m2 will be varied. The acceleration vs. (m1-m2) graph will be used to determine g (from the slope) and f (y-intercept)

PROCEDURE:

- Attach the Smart Pulley to the Photogate using the included rod. Thread the string around the pulley. Connect the Smart Pulley to the Dig/Sonic1 port of the Lab Pro interface.

- Attach the rod to a lab stand and secure the lab stand to the table with a C clamp.

- Start LoggerPro and open Experiment/Probes and Sensors/Photogate/Pulley.cmbl.

- Attach 2 hangers on the 2 ends of the string. Add slotted weights (450 g) on each hanger, using many small weights (2 g) on one of the hangers.  During the experiment, the total mass will remain constant but the mass difference m1-m2 will be changed by moving small weights from one hanger to the other.

- Since there are 450 g on each hanger, your system is now in equilibrium. To estimate the friction force, move small weights from one hanger to the other until the system barely starts moving. Record the mass difference at which the system starts moving. This difference in weight represents the force needed to overcome the friction. Record this value below:

f = ________ N

- Restore the equilibrium (450 g on each hanger). Move 4 g from one hanger to the other, but keep the system from moving. The mass difference is now 8g. Record the mass difference in the table 9including units). Push the ►Collect button and release the system.

- Find a linear fit for the velocity graph. What is the significance of the slope of this graph? Record the value of the acceleration, including units, in the table.

- Repeat steps 6-7 for 8 more values of the mass difference m1-m2

- After all the values have been recorded, plot acceleration vs. m1-m2 graph using Graphical Analysis.  Find a best fit equation for your graph.

- Cut-and-paste the graph, including the best fit information, into Word.  Print out the graphs.

| Data Table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Trial # |  |  |  |  |  |  |  |  |  |
| m1-m2 |  |  |  |  |  |  |  |  |  |
| a |  |  |  |  |  |  |  |  |  |

ANALYSIS QUESTIONS

Refer to equation (1) to answer the following questions:

- What is the significance of the slope of the acceleration vs. m1-m2 graph? What are its units?

- From the slope of the acceleration vs. m1-m2 graph, determine the acceleration due to gravity, g. (don’t forget to take into account the mass of the hanger!!!). Enter your result below

g=_________

- Compare your value above to the accepted value and calculate the % difference. List several reasons for the % difference.

- Does the friction force account for the % difference calculated above? Why or why not? Be specific.

- What is the physical significance of the y intercept (b) of your fit equation? Calculate the friction force f using this coefficient and enter your result below.

f=_________.

- Compare your answer in (5) to the measured value of the friction force recorded above and calculate the % difference. List several reasons for the difference you observed.