## Experiment: Heat Energy Transfer

Purpose:

  - Compare the effectiveness of 3 heat transfer mechanisms

  - Verify the relationship between the rate of the heat transfer and temperature difference

Preliminary Questions:

1) During the spring, the outside temperature is (in average) 15 degrees Celsius, and during the winter it is 5 degrees Celsius. Compare the energy costs to maintain an inside temperature of 25 degrees Celsius, assuming that the price of energy does not change.

2) Consider 2 identical cups containing the same amount of water at the same temperature of 45 degrees Celsius, one of them covered and the other one not. After 5 minutes, would you expect the temperature to be the same in the 2 cups? Why?

3) Two identical closed metallic cups containing water are exposed to light from a lamp. One of the cups is painted black, the other one is white.

a)	Will the temperature of the water change? Why?

b)	Which cup will have a higher temperature after 5 minutes?

c)	If the 2 cups contain hot water (at 85 degrees Celsius) which one will cool down first? Why?

Introduction:

The transfer of heat energy from a cup containing hot water to its surroundings takes place through several different mechanisms. There is conduction through the cup into the table and the surrounding air. There is convection from the air flow that results when the air is heated by the water and cup. Finally, there is radiation the electromagnetic waves (mostly infrared) emitted from the hot surfaces of the cup.

The rate of the heat transfer by conduction depends on the transfer surface, the length and the temperature difference between the cup and the surroundings:

Equation 1:

The rate of the heat transfer by radiation depends on the temperature, area of  the surface and the emissive properties of the cup:

Equation 2:

The heat absorbed/released will be calculated from:

Equation 3:

The temperature change and the mass will be measured during the experiment

Materials:

  - LabPro interface & computer

  - LoggerPro software

  - Styrofoam Cups

  - Electric immersion heater

  - Heat lamp

  - Black and shiny identical containers

  - 2 temperature sensors

### Experiment:

Part I: Heat transfer through convection

- Plug two temperature sensors into the LabPro interface (Channel 1 and 2, respectively).

- Open the LoggerPro software. In LoggerPro, open the experiment file: Experiments/Additional Physics/Real Time Physics/Heat and Thermodynamics/Cooling Down (L3A2-1)

- Record the room air temperature

- Using an immersion heater, warm ~200 mL water in a beaker to 90oC. Quickly but carefully, pour a 75 ml water sample into two separate Styrofoam cups. Place a temperature sensor into each cup. Cover one of the cups but leave the other uncovered.  Read the initial temperature of the water (around 85oC). Push “Collect” to start graphing.

- Store this run.

- Using the Examine/Analyze feature, record the initial and the final temperature for each graph. Record data in Table 1.

- Cut-and-paste the graphs into Microsoft WORD and print.

QUESTION 1: What is the heat transfer mechanism(s) for the covered cup? For the uncovered cup?

QUESTION 2: Calculate the heat lost by the covered and uncovered cup, respectively. What is the rate of heat transfer for the 2 cases?

### Part II: Heat transfer through conduction

- Record room air temperature

- Heat water in a beaker to 70oC. Quickly pour 75 ml water into a same Styrofoam cup. Put the temperature sensor in the cup and cover the cup. Read the initial temperature of the water (around 65oC). Push Collect to start graphing.

- Store this run.

- Repeat steps 1-3 with water at the initial temperature of 45 oC.

- Cut-and-paste the graphs into Microsoft WORD and print.

- Calculate for each case the heat energy transferred and the rate of the heat energy transfer.

QUESTION 3: Explain how your data (or not) using equation 1.

QUESTION 4: Compare the effectiveness of convection vs. conduction as mechanisms of heat transfer.

Mass of water =_________	{should be the same for each of these trials}

| Table 1 | TInitial | TFinal | T | Qlost |  | Heat transfer mechanism |
| --- | --- | --- | --- | --- | --- | --- |
| Cup covered |  |  |  |  |  |  |
| Cup uncovered |  |  |  |  |  |  |
| Cup covered |  |  |  |  |  |  |
| Cup covered |  |  |  |  |  |  |

### Part III: Heat transfer through radiation

1) Open the experiment file called Radiation (L3A3-2).

2) Set a heat lamp 35 cm in front of a can, at the same height; the face of the lamp and the can should be directly across.

3) Pour 75 ml of water (room temperature) into a black can. Place the cover tight on the can.  Insert the temperature sensor through the cover.  Place a piece of paper on the temperature sensor, so that it prevents radiation from lamp from reaching the temperature sensor directly.

4) Record the initial temperature of the water in Table 2. Begin graphing and turn on the lamp.  Gently stir the water while collecting data.

5) When the data collection is over, record the final temperature. Store the run. Turn off the lamp.

6) Repeat steps 2-5 with an unpainted, shiny cup (PLACED IN THE SAME EXACT PLACE AS THE BLACK CAN). Record the temperatures.

7) Cut-and-paste the graphs into Microsoft WORD and print.

| Table 2 | TInitial | TFinal | T | Qgained |
| --- | --- | --- | --- | --- |
| Black can |  |  |  |  |
| Shiny can |  |  |  |  |

QUESTION 5: Which cup received more heat? Calculate the ratio Qblack/Qshiny.

QUESTION 6: Based on your results in Table 2, which can will lose heat by radiation faster?

8) To test your prediction (question 6) pour 75 ml hot water (80oC) in the black can. Record the initial temperature. Collect data for 5 min. Record the final temperature. Repeat step 7 with the shiny can.

| Table 3 | TInitial | TFinal | T | Qlost |
| --- | --- | --- | --- | --- |
| Black can |  |  |  |  |
| Shiny can |  |  |  |  |

QUESTION 7: Which can lost more heat?