Experiment: Ohm’s Law

objectives

- Determine the mathematical relationship between current, potential difference, and resistance in a simple circuit.

- Compare the potential vs. current behavior of a resistor to that of a light bulb.

- Estimate the temperature of the filament of a glowing light bulb.

MATERIALS

| - Windows PC | - wires |
| --- | --- |
| - LabPro Interface | - clips to hold wires |
| - Logger Pro | - switch |
| - Current Probe | - two resistors (~ 10 and 50 W) |
| - Differential Voltage Probe | - a small light bulb (6.3 V) |
| - Analog Out connector |  |

The fundamental relationship among the three important electrical quantities current, voltage, and resistance was discovered by Georg Simon Ohm. The relationship and the unit of electrical resistance were both named for him to commemorate this contribution to physics. One statement of Ohm’s law is that the current through a resistor is proportional to the voltage across the resistor. In this experiment you will test the correctness of this law in several different circuits using a Current & Voltage Probe System and a computer.

These electrical quantities can be difficult to understand, because they cannot be observed directly. To clarify these terms, some people make the comparison between electrical circuits and water flowing in pipes. Here is a chart of the three electrical units we will study in this experiment.

| Electrical Quantity | Description | Units | Water Analogy |
| --- | --- | --- | --- |
| Voltage or Potential Difference | A measure of the Energy difference per unit charge between two points in a circuit. | Volts (V) | Water Pressure |
| Current | A measure of the flow of charge in a circuit. | Amperes (A) | Amount of water flowing |
| Resistance | A measure of how difficult it is for current to flow in a circuit. | Ohms (W) | A measure of how difficult it is for water to flow in a pipe. |

QUESTIONS

1.  Consider a simple circuit with a 100  resistor connected in series with an adjustable voltage power supply.  A voltage and current probe are connected to this circuit to measure the potential difference across the resistor and the current through the resistor.

a.	Sketch a diagram for this circuit including the voltage and current probes.

b.	How much current flows through the resistor when the voltage of the power supply is 12 V?

c.	If the voltage were to double, how would you expect the current to change?

2.	A small light bulb is connected to the simple circuit above, replacing the resistor.  At 20oC, the resistance of the tungsten filament in the bulb is 3.3 .  Note: the bulb filament is non-Ohmic.

a.	How much current flows through the resistor when the voltage of the power supply is set to 0.2 V and the bulb filament temperature is 20oC?

b.	Based on the resistivity of tungsten, how would you expect the resistance of the filament to change when it operates at a temperature of 1500oC?

c.	How much current flows through this bulb when the power supply is set to 12V, and the bulb temperature is 1500oC?

d.	What is the power output of this bulb in c?

´

Figure 1

PRELIMINARY SEtup

1.	Connect the Voltage Probe to CH1 and the Current Probe to CH2 of the LabPro interface.  Connect the Analog Out connector to Ch4 (this will be your power supply).

2.	Start the LoggerPro software then open the experiment file: “Exp 25” from the Physics with Computers experiment files.

3.	Click on the LabPro icon (upper left hand side of the screen) and select Ch4. Select “Analog Out” then perform the following:

- Set the Waveform to “DC Output”

- Set the Amplitude to 0 and the Increment to 0.2

- Click on the “Create User Parameters” box

- Click OK

4.	With the power supply turned off, connect the Analog Output connector (i.e. power supply), 10-W resistor, wires, and clips as shown in Figure 1. Take care that the positive lead from the power supply and the red terminal from the Current & Voltage Probe are connected as shown in Figure 1. Note: Attach the red connectors electrically closer to the positive side (white wire) of the Analog Out cable.

5.	Click . A dialog box will appear. Click . This sets the zero for both probes with no current flowing and with no voltage applied.

6.	Have your instructor check the arrangement of the wires before proceeding. Turn the control on the DC power supply to 0 V and then turn on the power supply. Slowly increase the voltage to 4 V. Monitor the Meter window in Logger Pro and describe what happens to the current through the resistor as the potential difference across the resistor changes.

Question: If the voltage doubles, what happens to the current?

Question: What type of relationship do you believe exists between voltage and current?

PROCEDURE

1.	Record the value of the resistor in the data table.

2.	Make sure the Amplitude is set to 0 V. Begin data collection. Monitor the voltage and current. When the readings are stable click .

3.	Increase the Amplitude to 0.2 (approximately 0.2 V). When the readings are stable click . Repeat this process, in 0.2 Amplitude steps, until you reach a voltage of 4.0 V.

4.	When data collection is complete, set the Amplitude back to 0.

5.	Are the voltage and current proportional? Fit the graph to a linear function (linear regression) and record the slope and y-intercept of the regression line, including the standard deviations (along with their units).

7.	Cut-and-paste the graph into a Word document.

8.	Repeat Steps 1–7 using a different resistor.

9.	Replace the resistor in the circuit with a small light bulb. Repeat Steps 2 – 5, but this time increase the voltage in 0.1 Amplitude steps up to 2.2.  You can go higher as long as the measured voltage continues to increase and the current does not exceed 0.5 mA.

10.	Fit the first 3 to 5 data points (only over a “linear” region of the left side of the graph) to a linear function.  Record the slope and standard deviation of the linear fit (including units).  This the resistance of the bulb near 20oC.

11.	Fit the last 3 to 5 points on the graph (again, only over a “linear” region of the graph) to a linear function. Record the slope and standard deviation of the regression line in the data table.  This the resistance of the bulb at “peak” temperature.

12.	Cut-and-paste the graph into a Word document. Print all of the graphs together.

DATA TABLE

|  | Fit Slope | slope | y-int | y-int |
| --- | --- | --- | --- | --- |
| Resistor W |  |  |  |  |
| Resistor W |  |  |  |  |
| Light bulb (first 3 pts) |  |  |  |  |
| Light bulb (last 10 pts) |  |  |  |  |

ANALYSIS

1.	Compare the slope value (from the V vs i graph) for each resistor to the labeled resistance value. Calculate % Error.

2.	Resistors are manufactured such that their actual value is within a “tolerance”.  For most resistors used in this lab, the tolerance is 5% or 10%.  Calculate the range of values for each resistor (based on the tolerance). Does the slope of V vs. i for each resistor (including the slope uncertainty) fit within the appropriate range of values for the corresponding resistor?

3.	Do your resistors follow Ohm’s law? Base your answer on your experimental data.

4.	Describe what happened to the current through the light bulb as the potential increased.

i.	Was the change linear? Since the slope of the linear regression line is a measure of resistance, describe what happened to the resistance as the voltage increased.

ii. Since the bulb gets brighter as it gets hotter, how does the resistance vary with temperature?

5.	The relationship between T and R is approximately:

R = Ro(1 + (T-To))

where the temperature coefficient of resistivity () for tungsten is 4.5x10-3/oC.

Using this equation, estimate the temperature of the filament for the largest R value measured for the light bulb.

6.  According to many textbooks, the temperature of a tungsten filament in a glowing bulb is about 1700K.  Compare your temperature estimate to this value and calculate the % error.